# AI Agents + Revit via MCP: Student Instructions

# Goal

Connect an AI agent (Muse from Meta) to your own copy of Revit through MCP (Model Context Protocol), then have the agent build a small BIM model from tutorial slides while you supervise. You will see:

- how an AI agent operates real software through tools, not only through chat;

- what the setup involves;

- where the agent succeeds, where it fails, and how to find the cause of a failure.

# What you need (before class)

| What | Where to get it |
| --- | --- |
| A Windows PC with Revit 2026, on which you have administrator rights | Free with an Autodesk Education account: autodesk.com/education |
| Tailscale (free), installed and signed in on that PC | Windows installer (Windows 10 or later): tailscale.com/download/windows Account: login.tailscale.com/start |
| The Muse from Meta app (free, with a usage limit), signed in | iPhone (iOS 18 or later): App Store Android: Google Play All versions: ai.meta.com/muse/download |
| The Revit MCP plug-in (mcp-servers-for-revit, Revit 2026 version) | The zip file from your instructor. Do not use another download. |

# Setup

- Start Tailscale on your PC and check that it shows you as signed in (the Tailscale icon in the taskbar).

- Install the plug-in: close Revit, unzip the plug-in file, and copy mcp-servers-for-revit.addin and the revit_mcp_plugin folder into:

C:\ProgramData\Autodesk\Revit\Addins\2026\

Paste this path into the File Explorer address bar (the ProgramData folder is hidden otherwise). Windows asks for administrator permission to copy into this folder.

- Open Revit, start a new project from the default template, and click Always Load if Revit asks about the add-in.

- Enable the commands: Revit MCP tab → Settings → select RevitMCPCommandSet → Select All → Save. Then turn the Revit MCP Switch on. The commands load only when the switch is turned on: if Muse reports "method not found," repeat this step.

- Let Muse in: paste the initial prompt below into Muse. Muse sends you a Tailscale link: approve its device ("muse") joining your network.

- Check the connection before building: Muse first makes read-only calls (your current view, your model's element statistics). Compare them with your screen before you approve any change.

Security note: the plug-in has no password, so any device on your Tailscale network can reach Revit. Approve only devices you recognize, and when you finish, turn the Revit MCP Switch off or quit Tailscale.

# Initial prompt (paste into Muse)

Set up an MCP connection to Revit 2026 on my Windows PC. I have installed Tailscale and the revit-mcp plug-in, enabled the RevitMCPCommandSet, and turned the Revit MCP Switch on. Join my Tailscale network so you can reach my PC, then verify the connection with read-only calls only: tell me my current view name and my model's element statistics. Do not create or change anything until I approve it. After I confirm the reads look right, I will give you tutorial slides to carry out step by step. Flag any step you have to approximate or cannot do, and say why.

# During the exercise

- Send Muse the tutorial slides (PDF) one section at a time, and approve each write action when Muse asks.

- Check your screen after each step; do not rely on the agent's report alone.

- Expect some steps to fail. Note which step failed, what Muse reported, and what you saw in Revit.

- Ctrl+Z in Revit undoes the agent's actions, the same as your own.